
Appendix D
Part 445-2-H Chapter 16


SNOWMOBILE PREVENTATIVE

MAINTENANCE CHECKS AND SERVICES

Pre-mission preventive maintenance checks and services safety inspections of snowmobiles are important to minimize the risks of injuries, breakdowns, or being stranded.
Operator___________________________________Date_____________ 

Supervisor__________________________________Date ____________ 

Safety Inspection Procedures

Track and Skis

1.  Check track for any wear or tear.
2.  Check track for proper alignment and adjustments.
3.  Check between track for any obstructions (ice, rocks, or other debris).

4.  Check traction control devices for damage, wear, or if missing.
5.  Check skis for proper alignment and wear.
6.  Check ski wearbars for wear and replace if worn.

Controls, Cables, and Belts

1.  Check all controls and cables to ensure that they are working properly.
2.  Make sure throttle moves smoothly and snaps closed.
3.  Check brakes, parking brake levers, and cables.
4.  Check fan belt (if fan cooled).

5.  Check steering linkage and handlebar position. 

Lights and Electrical System

1.  Check ignition switch, engine stop switch, tether switch, and AC outlets (if equipped).
2.  Check headlamp (high/low beams, tall lamp, and brake lamp).
3.  Check battery, fluid levels, and for terminal corrosion.
4.  Check electric start switch. 

Oil, Fuel, and Coolant

1.  Check injection oil, engine oil, and chain drive oil levels.
2.  Check fuel level.
3.  Check coolant fluid level.
4.  Check for any fuel, oil, and coolant leaks and properly fill all fluid levels. See owner’s manual.
Suspension, Drive Shaft, Chain, and Chassis

1. Check front and rear suspension for travel and compression. Clean of any debris.
2.  Check front and rear suspension springs and shocks for loose or worn parts.
3.  Check chain, lubricate, and adjust.  See owner’s manual.
4.  Check drive shaft, front and rear.  Inspect for damage, bent, or loose.
5.  Check chassis for any loose, worn, or missing parts.
6.  Check footrests (secure and clean of debris). 

Muffler/Exhaust System

1.  Check muffler and exhaust system (loose, cracked, missing springs, or exhaust leaks).
2.  Check exhaust and heat shields and tighten if loose.

Tool Kit, Spare Parts, and Owner’s Manual

1.  Tool kit shall include the following:  spark plug wrench, pliers, wrenches, screwdrivers, etc.
2.  Spare parts shall include:  spark plugs, plastic wire ties, duct tape, etc.

3.  Owner’s manual shall be readily available.
