U.S. Geological Survey Manual

Appendix A

Part 445-2-H Chapter 41

Cableway Visual Inspection Checklist

Station Name/Number_____________________________________

Date of last cableway inspection_____________ (Do not use the cableway if it has been more than 1 year since last inspection.)

Design unloaded sag for this cable is _______ feet.

Maximum stage for use of this cableway is ______ feet.

Before using the cableway at this station, please perform the following to the extent possible:

1.  Visually check all support devices (the A-frames/towers, cables, cable car, and all connections) for any signs of tampering, vandalism, or deterioration.

2.  Visually check the main cable and the cable clips (# of clips: ___) for proper installation, tightness, or deterioration (rust, corrosion, etc.).

3.  Visually check the backstay cables or guy lines which secure the A-frames/towers in the upright position.

Make sure thimbles and the cable clips (# of clips: ___) are installed properly.

4.  Visually check footings, anchors, and U-bar areas. Make sure all connections and fasteners are not buried, loose, bent, badly corroded, or showing signs of metal fatigue.

5.  Visually check cable car for loose, badly corroded, bent or broken members.  Verify that the car meets U.S. Geological Survey standards (Hydrologic Instrumentation Facility [HIF] cable cars with reinforcing members; or non-HIF cable cars tested for compliance).  Make sure that puller, car braking system, sheaves, etc., are in proper working order.

Caution: Do not use the cableway if any deficiencies are found.

Note: The deficiencies on an inspection checklist and submit the list to your supervisor for inclusion in the Site Information Management System.  Do not tighten the cable - reducing the sag - unless the sag diagram and a level are available, and an experienced person is involved.  (Check 1-4 above for possible causes of excess sag before adjusting.) Make sure every person in the cable car is wearing a personal floatation device.  Never put your hands on the cable when the car is moving.  Wear gloves.  Always carry a wire-cutting tool when conducting streamflow or other hydrologic measurements.